Université Antonine

Faculty of Engineering

Department of Computer and Communications Engineering

Midterm – S1 2026-2027

Distributed Systems

Distributed Object-Tolerant Storage Framework

- You must develop a distributed framework that splits files into content-addressed chunks, distributes them across multiple storage nodes, keeps them available through replication and erasure coding, and exposes them through an S3-like API.

- At the end of the two sessions, you must submit a document describing your work.

- You must test your framework on two different workloads: (a) many small files (for example, 5,000 files of 1–50 KB, such as text or JSON) and (b) a few large files (for example, 10 files of 100 MB–1 GB, such as video or disk images).

- You must submit a detailed report that includes full-screen screenshots for each phase of the project, each accompanied by a paragraph explaining the steps taken in that phase. Additionally, the report must contain a flowchart illustrating each phase of the framework (write path, read path, failure and repair path) to provide a clear visual representation of the system’s workflow.

- You must deliver all files created, used, or included (source code, Dockerfiles, configuration files, test scripts, datasets or the scripts that generate them, and benchmark results).

Objectives

- Implement a scalable storage layer distributed over at least 4 storage nodes.

- Implement data placement using consistent hashing so that adding or removing a node moves only a fraction of the data.

- Ensure fault tolerance through replication and erasure coding, with automatic detection of failed nodes and repair of lost chunks.

- Ensure data integrity through content addressing (SHA-256) and periodic scrubbing.

- Reduce storage cost through chunk-level deduplication.

- Provide APIs for storing, retrieving, listing and searching objects.

Tasks

1. Set up the environment

- Install Python and set up a virtual environment.

Université Antonine

Department of Computer and Communications Engineering

Midterm – S1 2026-2027

Distributed Systems

1. Set up the environment (continued)

- Set up a version control system (Git/GitHub/GitLab) and initialize a repository.

- Install Docker and Kubernetes.

- Allocate a separate data directory (or Docker volume) for each storage node.

2. Develop the Storage Node Module

- Use FastAPI (or gRPC) to build a storage node service exposing PUT, GET, DELETE and HEAD on chunks.

- Address every chunk by the SHA-256 hash of its content and store it on the node’s local disk.

- Verify the hash on every read and reject corrupted chunks.

- Send a heartbeat from each node to the metadata service every few seconds (node id, free space, chunk count).

- Run at least 4 storage nodes in separate containers.

3. Implement the Metadata and Placement Module

- Split incoming files into chunks: fixed-size (for example 4 MB) first, then content-defined chunking (rolling hash, for example FastCDC) for comparison.

- Store object metadata (bucket, key, size, version, ordered list of chunk hashes, creation date) in etcd or PostgreSQL.

- Implement a consistent hashing ring with virtual nodes to map each chunk hash to its target nodes.

- Implement chunk-level deduplication with reference counting, so identical chunks are stored once and deleted only when no object references them.

- Test placement by adding and removing a node and measuring the percentage of chunks that move.

4. Implement Redundancy and Recovery

- Implement replication with a configurable factor N (default 3) and quorum writes and reads (W + R > N).

- Implement erasure coding (Reed-Solomon, k data + m parity, for example 4+2) using zfec or pyeclib, selectable per bucket.